Memory Transfer

#include<reg51.h>

void main()

{

int source[4]= {0X10,0X20,0X30,0X40};

int destination[4];

int i;

for(i=0;i<4;i++)

{

destination[i]=source[i];

}

while(1);

}

LED

#include<reg51.h>

void delay(){

int i,j;

for(i=0;i<1000;i++)

{

for(j=0;j<100;j++);

}

}

void main()

{

int z;

while(1)

{

for(z=0;z<9;z++)

{

P1=z;

delay();

}

}

}

#include<reg51.h>

void delay()

{

int i,j;

for(i=0;i<1000;i++)

{

for(j=0;j<100;j++);

}

}

#include<reg51.h>

void delay()

{

int i,j;

for(i=0;i<1000;i++)

for(j=0;j<100;j++);

}

void main()

{

int x;

while(1)

{

for(x=0;x<16;x++)

{

P1=x;

delay();

}

}

}

void main()

{

char mynum[] = "0123456ABC";

int z;

while(1)

{

for(z=0;z<11;z++)

P1=mynum[z];

delay();

}

}

Seven segment 0-9

#include<reg51.h>

void display(char byte);

void delay();

void main()

{

P0=0x00;

P1=0x04;

while (1)

{

display(0xc0);

display(0xf9);

display(0xa4);

display(0xb0);

display(0x99);

display(0x92);

display(0x82);

display(0xf8);

display(0x80);

display(0x90);

}

}

void display(char byte)

{

P0=byte;

delay();

}

void delay()

{

int i,j;

for(i=0;i<100;i++)

{

for(j=0;j<100;j++);

}

}

Seven segment 0-99

#include<Reg51.h>

#define SegOne 0x02;

#define SegTwo 0x01;

void delay_ms(unsigned int ms_count)

{

unsigned int i,j;

for(i=0;i<ms_count;i++)

{

for(j=0;j<15;j++);

}

}

void main()

{

char seg_code[]={0xc0,0xf9,0xb0,0x99,0x92,0x82,0xf8,0x80,0x90};

int cnt,num,temp,i;

while(1)

{

for(cnt=0x00;cnt<=99;cnt++)

{

for(i=0;i<100;i++)

{

num=cnt;

temp=num%10;

P2=SegOne;

P1=seg_code[temp];

delay_ms(10);

temp=num/10;

P2=SegTwo;

P1=seg_code[temp];

delay_ms(10);

}

}

}

}

Stepper motor

#include <reg51.h>

sbit IN1 = P1^4;

sbit IN2 = P1^5;

sbit IN3 = P1^6;

sbit IN4 = P1^7;

//To enable L293D Motor driver   IC  EN1 & EN2

# define P1^0 1;

# define P1^1 1;

void delay();

void main()

{

while(1)

{

IN1 = 0; IN2 = 0; IN3 = 0; IN4 = 1;

delay();

IN1 = 0; IN2 = 0; IN3 = 1; IN4 = 0;

delay();

IN1 = 0; IN2 = 1; IN3 = 0; IN4 = 0;

delay();

IN1 = 1; IN2 = 0; IN3 = 0; IN4 = 0;

delay();

}

}

void delay()

{

unsigned int i, j;

for(i = 0; i < 10; i++)

for(j = 0; j < 100; j++);

}

Another way to perform Stepper Motor

#include <reg51.h>

void delay();

void main()

{

int source[4]= {0X13,0X23,0X43,0X83};

int i;

while(1)

{

for(i=0;i<4;i++)

{

P1=source[i];

delay();

}

}

}

void delay()

{

unsigned int i, j;

for(i = 0; i < 10; i++)

for(j = 0; j < 100; j++);

}

#include <reg51.h>

void delay();

void main()

{

int source[4]= {0X13,0X23,0X43,0X83};

int i;

while(1)

{

for(i=0;i<4;i++)

{

P1=source[i];

delay();

}

}

}

void delay()

{

unsigned int i, j;

for(i = 0; i < 10; i++)

for(j = 0; j < 100; j++);

}

DAC Square wave

#include<reg51.h>

void delay()

{

TMOD=0X10;

TH1=0XFF;

TL1=0X1A;

TR1=1;

while(TF1==0);

{

TR1=0;

TF1=0;

}

}

void main()

{

while(1)

{

P1=0x00;

delay();

P1=0xff;

delay();

}

}

DAC sawtooth

#include<reg51.h>

void delay(){

TMOD=0X10;

TH1=0XFF;

TL1=0X1A;

TR1=1;

while(TF1==0);

TR1=0;

TF1=0;

}

void main()

{

unsigned char value=0;

while(1)

{

For(value=0;value<32;value++)

{

P1=value;

delay_timer1;

}

}

}

Traingular Wave

#include<reg51.h>

void delay()

{

TMOD=0X10;

TH1=0XFF;

TL1=0X1A;

TR1=1;

while(TF1==0);

{

TR1=0;

TF1=0;

}

}

void main()

{

unsigned char i;

while(1)

{

P1=0X00;

for(i=0;i<255;i++)

{

P1=i;

delay();

}

for(i=255;i>0;i--)

{

P1=i;

delay();

}

}

}

another way to perform stepper motor

#include <reg51.h>

void delay()

{

int i,j;

for(i=0;i<10;i++)

for(j=0;j<100;j++);

}

void main() {

while (1) {

P1 = 0x03;

delay();

P1 = 0x06;

delay();

P1 = 0x0C;

delay();

P1 = 0x09;

delay();

}

}

PIC LED BUZZER

#include<p18f4520.h>

#pragma config OSC=HS

#pragma config PWRT=OFF

#pragma config WDT=OFF

#pragma config DEBUG=OFF, LVP=OFF

void msdelay(unsigned int itime);

void Right(void);

void Left(void);

#define Relay PORTDbits.RD2

#define BUZZER  PORTDbits.RD3

#define SW1   PORTDbits.RD0

#define SW2   PORTDbits.RD1

#define LD1   PORTDbits.RD4

#define LD2   PORTDbits.RD5

#define LD3   PORTDbits.RD6

#define LD4   PORTDbits.RD7

unsigned int i,data;

void main()

{

TRISD=0x03;

ADCON1=0x0F;

PORTD=0x00;

Relay=0;

BUZZER =0;

while(1)

{

if(SW1==0)

Left();

if(SW2==0)

Right();

}

}

void Right(void)

{

Relay=1; BUZZER =0;

while(SW1!=0)

{

if(SW1==0)

Left();

LD1=1;LD2=0;LD3=0;LD4=0;Relay=0;

msdelay(30);

LD1=0;LD2=1;LD3=0;LD4=0;Relay=0;

msdelay(30);

LD1=0;LD2=0;LD3=1;LD4=0;Relay=0;

msdelay(30);

LD1=0;LD2=0;LD3=0;LD4=1;Relay=0;

msdelay(30);

}

}

void Left(void)

{

Relay=0;

BUZZER =1;

while(SW2!=0)

{

if(SW2==0)

Right();

LD1=0;LD2=0;LD3=0;LD4=1;Relay=1;

msdelay(30);

LD1=0;LD2=0;LD3=1;LD4=0;Relay=1;

msdelay(30);

LD1=0;LD2=1;LD3=0;LD4=0;Relay=1;

msdelay(30);

LD1=1;LD2=0;LD3=0;LD4=0;Relay=1;

msdelay(30);

}

}

void msdelay (unsigned int itime)

{

int i,j;

for(i=0;i<itime;i++)

for(j=0;j<1275;j++);

}

Lcd Display

/*

Company Name:- Logsun Systems

Project     :- 8-Bit LCD

Connection Details:-

PORTB = LCD Data

PORTD = LCD Command

*/

#include <P18f4520.h>

#pragma config OSC=HS

#pragma config PWRT=OFF

#pragma config WDT=OFF

#pragma config DEBUG=OFF, LVP=OFF

unsigned char text1[] = {" LOGSUN SYSTEMS"};

unsigned char text2[] = {"  PIC 18F4520"};

void delay(unsigned int value);

void lcdcmd(unsigned char value);

void lcddata(unsigned char value);

void lcdinit(void);

void lcddisplay(int row,unsigned char *str);

#define ldata PORTB

#define rs PORTDbits.RD3

#define rw PORTDbits.RD4

#define en PORTDbits.RD5

void main()

{

TRISD =0x00;

TRISB=0x00;

//	ADCON1=0x0F;

lcdinit();

lcddisplay(1,text1);

lcddisplay(2,text2);

while(1);

}

void lcddisplay(int row,unsigned char *str)

{

int k;

if (row==1)

lcdcmd(0x80);

else lcdcmd(0xC0);

for(k=0;k<16;k++)

{

if(str[k] !=0)

lcddata(str[k]);

else  break;

}

while((k<16)){

lcddata(' ');

k++;

}

}

void delay(unsigned int value)

{

int i,j;

for(i=0;i<=value;i++)

for(j=0;j<=50;j++);

}

void lcdcmd (unsigned char value)

{

ldata=value;

rs=0;

rw=0;

en=1;

delay(1);

en=0;

}

void lcddata (unsigned char value)

{

ldata=value;

rs=1;

rw=0;

en=1;

delay(1);

en=0;

}

void lcdinit(void)

{

lcdcmd(0x38);

delay(1);

lcdcmd(0x0E);

delay(1);

lcdcmd(0x01);

delay(1);

lcdcmd(0x06);

delay(1);

}

Timer With interrupt

#include<p18f4520.h>

#pragma config OSC=HS

#pragma config PWRT=OFF

#pragma config WDT=OFF

#pragma config DEBUG=OFF, LVP=OFF

void timer_isr(void);

#pragma code _HIGH_INTERRUPT_VECTOR = 0x08

void high_ISR (void)

{

_asm goto timer_isr _endasm    //The program is relocated to execute the interrupt routine timer_isr

}

#pragma code

// This function is executed as soon as the timer interrupt is generated due to timer overflow

#pragma interrupt timer_isr

void timer_isr(void)

{

TMR0H = 0X3C;              // Reloading the timer values after overflow

TMR0L = 0XB0;

PORTD = ~PORTD;              //Toggle the PORTD led outputs

INTCONbits.TMR0IF = 0;//Resetting the timer overflow interrupt flag

}

void main()

{

ADCON1 = 0x0F;        //Configuring the PORTE pins as digital I/O

TRISD = 0x00;                  //Configuring the LED port pins as outputs

PORTD = 0xFF;                //Setting the initial value of the LED's after reset

T0CON = 0x08;    //Set the timer to 16-bit mode,internal instruction cycle clock,no            prescaler

TMR0H = 0x3C;

TMR0L = 0xB0;

INTCONbits.TMR0IF = 0;      // Clear Timer0 overflow flag

INTCONbits.TMR0IE = 1;      // TMR0 interrupt enabled

T0CONbits.TMR0ON = 1;       // Start timer0

INTCONbits.GIE = 1;               // Global interrupt enabled

while(1);      //Program execution stays here untill the timer overflow interrupt is generated

}

Serial Communications

#include <p18F4520.h>

//Configuration bit setting//

#pragma config OSC=HS

#pragma config PWRT=OFF

#pragma config WDT=OFF

#pragma config DEBUG=OFF, LVP=OFF

#define _XTAL_FREQ 20000000

void Transmit_String(unsigned char *string)

{

unsigned char i=0;

for(i=0;string[i]!='\0';i++) //loop till end of the string

{

while(PIR1bits.TXIF == 0);

TXREG = string[i];

}

}

unsigned char MSG1[] = {"UART COMMUNICATION \r\n"};

unsigned char MSG2[] = {"Received Data \r\n"};

unsigned char MSG3[] = {" \r\n"};

void main(void)

{

unsigned char j=0;

unsigned char RX_DATA[10];

TRISCbits.TRISC7 = 1;   // RXD as Input

TRISCbits.TRISC6 = 0;   // TXD as Output

RCSTA = 0x90;

TXSTA = 0x24;

BAUDCON = 0x08;

SPBRG = 0x0A;

SPBRGH = 0x02;

Transmit_String (MSG1);

for (j=0; j<6 ; j++)

{

while(PIR1bits.RCIF == 0);

RX_DATA[j] = RCREG;

}

RX_DATA[6] = '\0';

Transmit_String (MSG2);

Transmit_String (RX_DATA);

Transmit_String (MSG3);

while(1);

}

//**************************************************************

//  Program for PWM Generation using PIC18F4520.

//  PWM output  	:       RC1

//**************************************************************

#include <p18f4520.h>

//#pragma config OSC = HS //Oscillator Selection

//#pragma config WDT = OFF //Disable Watchdog timer

//#pragma config LVP = OFF //Disable Low Voltage Programming

//#pragma config PBADEN = OFF //Disable PORTB Analog inputs

void myMsDelay (unsigned int time) 	// Definition of delay subroutine

{

unsigned int i, j;

for (i = 0; i < time; i++)		// Loop for i time

for (j = 0; j < 275; j++);	// Calibrated for a 1 ms delay in MPLAB

}

void main()

{

TRISCbits.TRISC4 = 0 ;	// Set PORTC, RC4 as output (DCM IN1)

TRISCbits.TRISC5 = 0 ;	// Set PORTC, RC5 as output (DCM IN2)

TRISCbits.TRISC1 = 0 ;	// Set PORTC, RC1 as output (CCP2 - DCM EN1)

PR2 = 0x4E;             // set PWM Frequency 4KHz

CCP1CON = 0x0C;         // Configure CCP1CON as PWM mode.

T2CON = 0x07;		//Start timer 2 with prescaler 1:16

PORTCbits.RC4 = 1;	// Turn ON the Motor

PORTCbits.RC5 = 0;

while(1)	// Endless Loop

{

// ----------Duty Cycle 25%-----------

CCP1CONbits.DC1B0 = 0;

CCP1CONbits.DC1B1 = 1;

CCPR1L = 0x13;

myMsDelay(2000);

//	 ----------Duty Cycle 50%-----------

CCP1CONbits.DC1B0 = 0;

CCP1CONbits.DC1B1 = 0;

CCPR1L = 0x27;

myMsDelay(2000);

//----------Duty Cycle 75%-----------

CCP1CONbits.DC1B0 = 0;

CCP1CONbits.DC1B1 = 1;

CCPR1L = 0x3A;

myMsDelay(2000);

}

}